Experiment / Assignment / Tutorial No.	4

Grade: AA / AB / BB / BC / CC / CD /DD

Signature of the Staff In-charge with date

Batch: A1 Roll No.: 1911004 Experiment / assignment / tutorial No.: 4

Title: 4 bit Magnitude Comparator

______________________________________________________________________________

Objective: Design a 2-bit comparator using logic gates and verify 4-bit magnitude comparator using IC 7485

_____________________________________________________________________________

Expected Outcome of Experiment:

CO2: Use different minimization technique and solve combinational circuits, synchronous & asynchronous sequential circuits.

______________________________________________________________________________

Books/ Journals/ Websites referred:

- VLab Link: http://vlabs.iitb.ac.in/vlabs-dev/labs/dldesignlab/experimentlist.html

- R. P. Jain, “Modern Digital Electronics”, Tata McGraw Hill

- M .Morris Mano, “Digital Logic & computer Design”, PHI

- http://elnsite.teilam.gr/ebooks/digital_design/lab/dataSheets_page/7485.pdf

Pre Lab/ Prior Concepts:

The comparison of two numbers is an operator that determines one number is greater than, less than (or) equal to the other number. A magnitude comparator is a combinational circuit that compares two numbers A and B and determines their relative magnitude. The outcome of the comparator is specified by three binary variables that indicate whether A>B, A=B (or) A<B.

Two Bit Magnitude Comparator Implementation Details:

Truth Table

| INPUT |  |  |  | OUTPUT |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | A0 | B1 | B0 | A > B | A = B | A < B |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 1 | 0 |

From the Truth Table:

(A<B) = A1’A0’B1’B0 + A1’A0’B1B0’ + A1’A0B1B0’ + A1’A0’B1B0 + A1’A0B1B0 + A1A0’B1B0
= (A1’A0’B1’B0 + A1’A0’B1B0) + (A1’A0’B1B0’ + A1’A0’B1B0) + (A1’A0B1B0’ + A1’A0B1B0) + (A1A0’B1B0 + A1’A0’B1B0)

= A1’A0’B0 + A1’A0’B1 + A1’A0B1 + A0’B1B0

= A1’A0’B0 + (A1’A0’B1 + A1’A0B1) + A0’B1B0

= A1’A0’B0 + A1’B1 + A0’B1B0

(A=B) = A1’A0’B1’B0’ + A1’A0B1’B0 + A1A0’B1B0’ + A1A0B1B0

(A>B) =  A1’A0B1’B0’ + A1A0’B1’B0’ + A1A0B1’B0’ + A1A0’B1’B0 + A1A0B1’B0 + A1A0B1B0’

= (A1’A0B1’B0’ + A1A0B1’B0’) + (A1A0’B1’B0’ + A1A0B1’B0’) + (A1A0B1’B0 + A1A0’B1’B0) + (A1A0B1B0’ + A1A0B1’B0’)

= A0B1’B0’ + A1B1’B0’ + A1B1’B0 + A1A0B0’ = A0B1’B0’ + (A1B1’B0’ + A1B1’B0) + A1A0B0’

= A0B1’B0’ + A1B1’ + A1A0B0’

Logic Diagram (2 bit Comparator)

Four Bit Magnitude Comparator Implementation Details

Pin Diagram of IC 7485

Logic Diagram of IC 7485

Comparing Truth Table

VLAB        (4-BIT COMPARATOR )

.

| INPUT - Input A : I/P15 to I/P12 - Input B : I/P9 to I/P6 |  |  |  |  |  |  |  | OUTPUT - A > B : O/P15 - A = B : O/P14 - A < B : O/P13 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  | B |  |  |  | A>B | A=B | A<B |  |
| I/P 15 | I/P 14 | I/P 13 | I/P 12 | I/P 9 | I/P 8 | I/P 7 | I/P 6 | O/P 15 | O/P 14 | O/P 13 |
| 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |

- A>B   (O/P 15=1 )

A=1000 B=0111

A=1001 B=0011

- A=B (O/P 14=1 )

A=1111 B=1111

A=0011 B=0011

- A<B (O/P 13=1 )

A=0011  B=1001

A=1100   B=1111

Conclusion:

We understood the concept of Comparator & their truth tables; 4-bit comparator was implemented in Virtual lab and necessary output was obtained.

Post Lab Descriptive Questions

  - Design a 1- bit magnitude comparator using logic gates.